RV-7 CONDITION INSPECTION REPORT

| OWNER | A/C TYPE | DATE |
| --- | --- | --- |
| SER. NO. | N NUMBER | MANUFACT. DATE |
| TYPE INSPECTION | TACH TIME | TOTAL A/C TIME |

| A. PROP. GROUP |  |  | ##### INSP. |  | O.K |
| --- | --- | --- | --- | --- | --- |
|  | 1 | INSPECT SPINNER AND BACKPLATE |  |  |  |
|  | 2 | INSPECT BLADES FOR NICKS AND CRACKS |  |  |  |
|  | 3 | INSPECT HUB FOR CRACKS AND CORROSION |  |  |  |
|  | 4 | CHECK FOR GREASE AND OIL LEAKS |  |  |  |
|  | 5 | CHECK MOUNTING BOLTS AND SAFETY WIRE (200RV – 40.4 ft-lb w/ 8 3/8” extension) |  |  |  |
|  | 6 | ###### CONSTANT SPEED (200 RV Propeller) |  |  |  |
|  |  | CHECK BLADES FOR TIGHTNESS IN HUB |  |  |  |
|  |  | LUBRICATE AS PER MANUAL |  |  |  |
|  |  | INSPECT COMPLETE ASSEMBLY |  |  |  |
|  | 7 | REPLACE SPINNER |  |  |  |
| B. ENGINE GROUP |  |  |  |  |  |
|  | 1 | REMOVE ENGINE COWLS |  |  |  |
|  | 2 | CLEAN COWLS, INSPECT FOR CRACKS, MISSING FASTENERS ETC. |  |  |  |
|  | 3 | ###### COMPRESSION TEST /80 HOT_______COLD______1_____2._____3._____4._____ |  |  |  |
|  | 4 | DRAIN OIL |  |  |  |
|  | 5 | CHECK OIL SCREENS AND CLEAN |  |  |  |
|  | 6 | REPLACE AND SAFETY OIL FILTER |  |  |  |
|  | 7 | CHECK OIL SENDER UNIT FOR LEAKS AND SECURITY |  |  |  |
|  | 8 | CLEAN AND CHECK OIL RADIATOR |  |  |  |
|  | 9 | REMOVE AND FLUSH OIL RADIATOR |  |  |  |
|  | 10 | CHECK AND CLEAN FUEL FILTER |  |  |  |
|  | 11 | SERVICE FUEL INJECTOR NOZZELS (60 in-lbs) |  |  |  |
|  | 12 | CHECK FUEL SYSTEM FOR LEAKS |  |  |  |
|  | 13 | CHECK FUEL LINES FOR LEAKS AND SECURITY |  |  |  |
|  | 14 | CHECK OIL LINES FOR LEAKS AND SECURITY |  |  |  |
|  | 15 | SERVICE AIR FILTER |  |  |  |
|  | 16 | CHECK HEATER AND CONTROLS |  |  |  |
|  | 17 | CHECK MAG POINTS |  |  |  |
|  | 18 | CHECK MAGS FOR OIL SEAL LEAKAGE |  |  |  |
|  | 19 | CHECK DISTRIBUTOR BLOCKS FOR ; CRACKS |  |  |  |
|  |  | :BURNED AREA |  |  |  |
|  |  | :CORROSION |  |  |  |
|  |  | HEIGHT OF CONTACT SPRINGS |  |  |  |
|  | 20 | CHECK MAG TIMING LEFT______RIGHT______ |  |  |  |
|  | 21 | CHECK IGNITION HARNESS AND INSULATORS |  |  |  |
|  | 22 | SERVICE OR REPLACE SPARK PLUGS (15 ft-lb or 180 in-lb) |  |  |  |
|  | 23 | CHECK CONDITION OF ALTERNATOR (13 ft-lb or 156 in-lb for belt tensioning) |  |  |  |
|  | 24 | CHECK CONDITION OF STARTER AND CONNECTIONS |  |  |  |
|  | 25 | INSPECT EXHAUST STACKS, GASKETS, NUTS, ETC. |  |  |  |
|  | 26 | INSPECT MUFFLERS, HEAT SHROUD |  |  |  |
|  | 27 | CHECK ENGINE BAFFLES |  |  |  |
|  | 28 | CHECK BREATHER TUBE OF OBSTRUCTIONS, SECURITY |  |  |  |
|  | 29 | CHECK CRANKCASE FOR LEAKS, CRACKS, FASTENERS, ETC. |  |  |  |
|  | 30 | CHECK ENGINE MOUNTS FOR CRACKS, LOOSE MOUNTS |  |  |  |
|  | 31 | CHECK ENGINE MOUNT BUSHINGS |  |  |  |
|  | 32 | CHECK FIREWALL SEALS |  |  |  |
|  | 33 | CHECK THROTTLE, MIXTURE, ALTERNATE AIR, PROP CONTROLS FOR OPERATION |  |  |  |
|  | 34 | INSPECT COWL EXHAUST TUNNEL FOR CRACKS AND CONDITION |  |  |  |
|  | 35 | INSPECT GENERAL CONDITION OF ENGINE |  |  |  |
|  | 36 | FILL ENGINE OIL..TYPE_________QUARTS________ |  |  |  |
|  | 37 | CLEAN ENGINE |  |  |  |
|  | 38 | LUBRICATE CONTROLS |  |  |  |
|  | 39 | SERVICE BATTERY |  |  |  |
| C. CABIN GROUP |  |  |  |  |  |
|  | 1. | INSPECT CANOPY HINGES, STRUTS, AND LATCHES |  |  |  |
|  | 2. | INSPECT CANOPY FRAME AND GLASS FOR GENERAL CONDITON |  |  |  |
|  | 3. | CHECK SEAT, SEAT BELTS, AND MOUNTINGS |  |  |  |
|  | 4. | CHECK TRIM OPERATION |  |  |  |
|  | 5. | CHECK FUEL ON-OFF VALVE FOR OPERATION |  |  |  |
|  | 6. | CHECK OPERATION OF FUEL DRAINS |  |  |  |
|  | 7. | CHECK OPERATION OF INTERIOR LIGHTING |  |  |  |
|  | 8. | CHECK RUDDER PEDALS AND CABLES |  |  |  |
|  | 9. | CHECK CONTROL STICK,BEARINGS, PUSHRODS FOR OPERATION AND SAFETY |  |  |  |
|  | 10 | CHECK INSTRUMENTS, LINES, WIRES |  |  |  |
|  | 11 | CHECK ALTIMETER CALIBRATION |  |  |  |
| ###### D. FUSELAGE GROUP |  |  |  |  |  |
|  | 1. | REMOVE ALL INSPECTION PANELS INCLUDING TAIL FAIRINGS |  |  |  |
|  | 2. | CHECK GENERAL CONDITION OF SKIN |  |  |  |
|  | 3. | REMOVE BAGGAGE COMPARTMENT COVERS AND BULKHEADS |  |  |  |
|  | 4. | CHECK WIRING, HOSES, ETC. FOR DAMAGE AND SECURITY |  |  |  |
|  | 5. | CHECK ANTENNA WIRES AND CONNECTIONS |  |  |  |
|  | 6. | CHECK AND SERVICE E.L.T. BATTERY GOOD UNTIL_________ (Test in first 5 min of hour) |  |  |  |
|  | 7. | CHECK BULKHEAD AND STRINGERS FOR DAMAGE |  |  |  |
|  | 8. | CHECK WING ATTACH BOLTS |  |  |  |
|  | 9. | CHECK EMPENNAGE SURFACES FOR DAMAGE |  |  |  |
|  | 10 | CHECK RUDDER HINGES, HORN, AND ATTACHMENTS |  |  |  |
|  | 11 | CHECK VERTICAL FIN ATTACHMENTS |  |  |  |
|  | 12 | CHECK ELEVATOR HINGES, HORN AND ATTACHMENTS |  |  |  |
|  | 13 | CHECK THAT ELEVATOR TRAVEL IS WITHIN LIMITS UP_______DOWN________ |  |  |  |
|  | 14 | CHECK HORIZONTAL STABILIZER ATTACHMENTS |  |  |  |
|  | 15 | CHECK HORIZONTAL TRIM FOR SECURITY AND OPERATION |  |  |  |
|  | 16 | CHECK OPERATION OF REAR STROBE AND POSITION LIGHT |  |  |  |
|  | 17 | RE-INSTALL PANELS AND TAIL FAIRING |  |  |  |
|  | 18 | GREASE AND SERVICE TAIL WHEEL |  |  |  |
| ###### E. WING GROUP |  |  |  |  |  |
|  | 1. | REMOVE INSPECTION PLATES AND FAIRINGS |  |  |  |
|  | 2. | CHECK SURFACE AND TIPS FOR DAMAGE |  |  |  |
|  | 3. | CHECK AILERON HINGES, PUSHRODS FOR OPERATION |  |  |  |
|  | 4. | CHECK AILERON TRAVEL IS WITHIN LIMITS RT UP_____RT DN______LT UP______LT DN______ |  |  |  |
|  | 5. | CHECK FLAP CONDITION AND HINGES |  |  |  |
|  | 6. | CHECK FLAP OPERATION AND MECHANISM |  |  |  |
|  | 7. | CHECK FLAP ALIGNMENT |  |  |  |
|  | 8. | CHECK WING ATTACH BOLTS |  |  |  |
|  | 9. | CHECK HEATED PITOT OF SECURITY, HOSE SECURITY, AND OPERATION |  |  |  |
|  | 10 | CHECK WINGTIPS FOR DAMAGE AND SECURITY |  |  |  |
|  | 11 | CHECK OPERATION OF LANDING LIGHTS |  |  |  |
|  | 12 | CHECK OPERATION OF TAXI LIGHTS |  |  |  |
|  | 13 | CHECK OPERATION OF STROBE LIGHTS |  |  |  |
|  | 14 | CHECK OPERATION OF POSITION LIGHTS |  |  |  |
|  | 15 | RE-INSTALL INSPECTION PANELS AND FAIRINGS |  |  |  |
| F. LANDING GEAR | 1. | PUT AIRCRAFT ON JACKS |  |  |  |
|  | 2. | REMOVE WHEEL PANTS AND GEAR FAIRINGS |  |  |  |
|  | 3. | CHECK TIRES FOR WEAR AND DAMAGE |  |  |  |
|  | 4. | REMOVE WHEELS INSPECT FOR CRACKS, CORROSION, CONDITION OF BOLTS |  |  |  |
|  | 5. | CHECK TIRE PRESSURES LEFT______ RIGHT______ |  |  |  |
|  | 6. | CHECK BRAKE LININGS AND DISCS |  |  |  |
|  | 7. | CHECK BRAKE LINES FOR LEAKS, SECURITY AND CONDITION |  |  |  |
|  | 8. | CHECK LANDING GEAR STRUTS FOR CRACKS AND SECURITY |  |  |  |
|  | 9. | CLEAN AND LUBRICATE MAIN WHEEL BEARINGS, AND RE-INSTALL MAIN WHEELS |  |  |  |
|  | 10 | RE-INSTALL BRAKES |  |  |  |
|  | 12 | CHECK BRAKE CYLINDERS FOR OPERATION AND LEAKS |  |  |  |
|  | 13 | CHECK BRAKE FLUID LEVEL. TYPE OF FLUID BEING USED: Mobile 1 ATF |  |  |  |
|  | 14 | INSTALL WHEEL PANTS AND GEAR FAIRINGS |  |  |  |
|  | 15 | REMOVE AIRCRAFT FROM JACKS |  |  |  |
| G. OPERATION CHECK |  |  |  |  |  |
|  | 1 | CHECK BOOST PUMP PRESSURE: ___________ |  |  |  |
|  | 2 | CHECK ALTERNATOR OUTPUT |  |  |  |
|  | 3 | CHECK AVIONIC EQUIPMENT OPERATION |  |  |  |
|  | 4 | CHECK OIL PRESSURE: ____________ |  |  |  |
|  | 5 | CHECK ENGINE FUEL PUMP PRESSURE |  |  |  |
|  | 6 | CHECK MANIFOLD PRESSURE |  |  |  |
|  | 7 | CHECK PROPELLER OPERATION AND SMOOTHNESS |  |  |  |
|  | 8 | CHECK MAG DROP AT 1800 RPM LEFT______RIGHT______ |  |  |  |
|  | 9 | CHECK STATIC R.P.M |  |  |  |
|  | 10 | CHECK IDLE R.P.M. |  |  |  |
|  | 11 | CHECK MAG SWITCH WIRES FOR GROUNDING |  |  |  |
|  | 12 | CHECK THROTTLE AND MIXTURE CONTROL OPERATION |  |  |  |
|  | 13 | CHECK IDLE MIXTURE |  |  |  |
|  | 14 | RE-INSTALL COWLS |  |  |  |
| H. GENERAL |  |  |  |  |  |
|  | 1. | AIRCRAFT CONFIRMS TO FAA SPECIFICATION |  |  |  |
|  | 2. | AIRCRAFT PAPERS IN PROPER ORDER A. REGISTRATION |  |  |  |
|  |  | B. AIRWORTHINESS |  |  |  |
|  |  | C. OPERATING LIMITS |  |  |  |

SIGNATURE OF INSPECTOR___________________________________________________________________

CERTIFICATE NUMBER________________________________________________________________________

DATE OF SIGN-OFF___________________________________________________________________________